GeODin Import Tool (GIT)

Bearbeiter:	Domen Mali

E-Mail:	d.mali@fugro.at

Folder: \\berlin5\hotline\8 - Projekte\AGS\04 - Tools\Universal Import Tool

Datum:	28.3.2013

| Date | Description |
| --- | --- |
| 18.02.2013 | Initial release |
| 06.03.2013 | AGS 3.1 and AGS 4.0 mapping update |
| 28.03.2013 | New Figures |

Table of Content

Features	4

Layout	4

Application menu	4

Options	5

Home	6

Data	6

Installation	7

Updating	8

# Features

- Importing AGS 3.1 and AGS 4.0 files with basic file structure validation

- Merging groups from different AGS 4.0 files into one AGS 4.0 file

- Editing values and store loaded data as GIT project

- Exporting into different formats:

- Microsoft Excel: exporting all groups into Microsoft Excel Spreadsheet

- Microsoft Access: exporting all groups into Microsoft Access database

- AGS 4.0: exporting back to AGS 4.0 file

- GeODin:

- Storing values for project, locations, layers, samples and CPT (G1 object type)

- Different standards for layer description (ASTMD2, BS5930)

- Converting units from SI system to US and vice versa

- Importing documents into document management (stored at project level)

- User friendly interface

- Advanced high performance DataGrid control for data shaping and data mining

- Wizard for easy usage

- Ability to change default application settings

- Three types of deployment:

- Windows Installer

- ClickOnce Installer

- ZIP-archive (network deployment)

# Layout

We try to adopt same principle as Microsoft Office applications using »ribbon« control. With this control we can easy extend application with additional functionalities (buttons) and still maintain user friendly GUI.

Ribbon tabs:

- Application menu

- Home

- Data

- About

# Application menu

Application menu offers user easy access to commands like creating new project or to change application settings.

Available commands:

- New: creates new GIT project

- Open: open existing GIT project

- Save: save GIT project

- Save as: save existing GIT project under different name

- Options: to change application settings

- Update: Manual check for updates

- About: Information about application

- Close: Exit from application

# Options

In application options user can set some of the settings.

General

In this section user can set if Wizard starts automatically at application start-up.

Updates

In this section user can set automatic update check at application start-up.

With this option enabled, application will check for update at every start-up. If new version is available, user will get a message with option to perform an update.

More about application update you will find in “Updating” section.

Note

In order you need Proxy to access the Internet, you need to set and save those settings in “Proxy” section of application options or perform update check manually.

Proxy

In this section user can set Proxy settings. These settings are used for application update.

Use Proxy: enable or disable Proxy

Proxy IP: Proxy IP address (only IP can be entered)

Port: Proxy port number

Use credential: Enable or disable Proxy authentication

Username: Proxy username

Password: Proxy password

Note:

If you don’t want to save Username and Password, uncheck “Use credentials”.

With “Set to default” button all modified settings are overwritten with default values.

# Home

Wizard

With multi-step wizard we can guide users thru different operations.

# Data

On Data page user can choose between different functionalities related to data manipulations (importing, merging, exporting, changing values …).

Import

For now we can import AGS 4.0 and AGS 3.1 files. During import basic file structure is verified (GROUP lines, number of columns …). For example: user is not able to import AGS 3.1 file, if AGS 4.0 file format is expected.

If something is wrong with file structure, user is notified with appropriate message.

Associated files

According to rule 20 of the AGS data format, each AGS file can contain additional files. Those files are defined inside FILE group and must be submitted along with AGS file in a standard file folder structure.

During AGS file import and/or merge this file structure is verified and in case of missing files user gets appropriate message. User also decides if missing files are removed from further processing or not.

In the current release of GIT all associated files are stored inside Microsoft Access database as binary data and can be accessed inside GeODin thru Document Management System.

Data Tools

Within this group we have functionalities related to data manipulation:

- Edit: to enable changing values inside Data Grid

- Merge: Merging AGS 4.0. files

AGS 4.0 file merging can be performed on selected files or on entire folder (without subfolders).

Export

Within Export group we can choose between different types of export:

- Export to Microsoft Excel: Exporting AGS file content to Microsoft Excel Spreadsheet

- Export to Microsoft Access: Exporting AGS file content into Microsoft Access database

- Export to GeODin: Before exporting to GeODin, user needs to specify:

- for which soil standard layers will be generated and

- in which unit system values should be stored.

- Export to AGS: Exporting to AGS 4.0 file. This option is only available for AGS 4.0 file type.

# Installation

GIT comes in three different deployment packages. To install or to use GIT, your system must meet the following requirements:

- Operating system Windows Server 2003/2008, Windows Vista or Windows XP

- Microsoft .NET Framework 3.5 SP1

Windows Installer

Windows Installer is a software component used for the installation, maintenance, and removal of software on Microsoft Windows systems.  This deployment should be used on computers where users have Administrator privileges.

Installation steps:

1. Download MSI file on local drive

2. Start installation and follow the installation steps

3. Start the program using desktop shortcut or navigate to Start -> Programs -> Fugro - > GeODin Import

ClickOnce Installer

ClickOnce is a deployment technology for creating self-updating Windows-based applications that can be installed and run with minimal user interaction.

Installation steps:

1. Download ZIP archive on local drive

2. Extract ZIP and

3. Start installation process by clicking on setup.exe and follow the installation steps

4. Start the program (Start -> Programs -> Fugro - > Universal Import Tool)

ZIP archive:

ZIP archive contains all files needed to run GIT on local or network drive. This package should be used on systems where user does not have Administrator privileges or for some reasons do not want to install it.

Installation steps:

1. Download ZIP archive on local drive

2. Extract ZIP

3. Start application with ImportTool.exe

# Updating

Updates are located on a remote server and therefore GIT application update can be performed over the Internet. To check for a new version go under “About” and click on “Update” button.

If a new version is available you will get a message:

Note:

After application update application will be automatically closed. To prevent any kind of data loss, please save all changes before you continue with the update process.